OrcaSlicer参数说明 副本

https://github.com/SoftFever/OrcaSlicer

## CLI

### 所有的option

| Option | Description | 中文翻译 |
| --- | --- | --- |
| --allow-multicolor-oneplate | If enabled, the arrange will allow multiple color on one plate | 如果启用，排列时将允许在一个板上使用多种颜色 |
| --allow-newer-file option | Allow 3mf with newer version to be sliced | 允许对更高版本的 3mf 文件进行切片 |
| --allow-rotations | If enabled, the arrange will allow rotations when place object | 如果启用，排列对象时将允许旋转 |
| --avoid-extrusion-cali-region | If enabled, the arrange will avoid extrusion calibrate region when place object | 如果启用，排列对象时将避开挤出校准区域 |
| --clone-objects "1,3,1,10" | Clone objects in the load list | 克隆加载列表中的对象 |
| --datadir | Load and store settings at the given directory. This is useful for maintaining different profiles or including configurations from a network storage. | 在指定目录加载和存储设置。这对于维护不同的配置文件或包含网络存储中的配置很有用。 |
| --debug level | Sets debug logging level. 0:fatal, 1:error, 2:warning, 3:info, 4:debug, 5:trace | 设置调试日志级别。0: 致命错误，1: 错误，2: 警告，3: 信息，4: 调试，5: 跟踪 |
| --downward-check | if enabled, check whether current machine downward compatible with the machines in the list | 如果启用，检查当前机器是否与列表中的机器向下兼容 |
| --downward-settings "machine1.json;machine2.json;..." | the machine settings list need to do downward checking | 需要进行向下兼容性检查的机器设置列表 |
| --enable-timelapse | If enabled, this slicing will be considered using timelapse | 如果启用，本次切片将考虑使用延时摄影功能 |
| --load-assemble-list assemble_list.json | Load assemble object list from config file | 从配置文件中加载组装对象列表 |
| --load-custom-gcodes custom_gcode_toolchange.json | Load custom gcode from json | 从 JSON 文件中加载自定义 G 代码 |
| --load-filament-ids "1,2,3,1" | Load filament ids for each object | 为每个对象加载丝材 ID |
| --load-filaments "filament1.json;filament2.json;..." | Load filament settings from the specified file list | 从指定的文件列表中加载丝材设置 |
| --load-settings "setting1.json;setting2.json" | Load process/machine settings from the specified file | 从指定文件中加载工艺/机器设置 |
| --makerlab-name name | MakerLab name to generate this 3mf | 生成此 3mf 文件的 MakerLab 名称 |
| --makerlab-version version | MakerLab version to generate this 3mf | 生成此 3mf 文件的 MakerLab 版本 |
| --metadata-name "name1;name2;..." | matadata name list added into 3mf | 添加到 3mf 文件中的元数据名称列表 |
| --metadata-value "value1;value2;..." | matadata value list added into 3mf | 添加到 3mf 文件中的元数据值列表 |
| --outputdir dir | Output directory for the exported files. | 导出文件的输出目录 |
| --skip-modified-gcodes option | Skip the modified gcodes in 3mf from Printer or filament Presets | 跳过 3mf 文件中来自打印机或丝材预设的已修改 G 代码 |
| --skip-objects "3,5,10,77" | Skip some objects in this print | 跳过本次打印中的某些对象 |
| --uptodate-filaments "filament1.json;filament2.json;..." | load uptodate filament settings from the specified file when using uptodate | 使用更新功能时，从指定文件中加载最新的丝材设置 |
| --uptodate-settings "setting1.json;setting2.json" | load uptodate process/machine settings from the specified file when using uptodate | 使用更新功能时，从指定文件中加载最新的工艺/机器设置 |
| --arrange option | Arrange options: 0-disable, 1-enable, others-auto | 排列选项：0 - 禁用，1 - 启用，其他 - 自动 |
| --assemble | Arrange the supplied models in a plate and merge them in a single model in order to perform actions once. | 将提供的模型排列在一个板上并合并为一个模型，以便一次性执行操作。 |
| --convert-unit | Convert the units of model | 转换模型的单位 |
| --ensure-on-bed | Lift the object above the bed when it is partially below. Disabled by default | 当对象部分位于工作台面以下时，将其提升到工作台面以上。默认禁用 |
| --orient | Orient options: 0-disable, 1-enable, others-auto | 定向选项：0 - 禁用，1 - 启用，其他 - 自动 |
| --repetitions count | Repetions count of the whole model | 整个模型的重复次数 |
| --rotate | Rotation angle around the Z axis in degrees. | 绕 Z 轴的旋转角度（度） |
| --rotate-x | Rotation angle around the X axis in degrees. | 绕 X 轴的旋转角度（度） |
| --rotate-y | Rotation angle around the Y axis in degrees. | 绕 Y 轴的旋转角度（度） |
| --scale factor | Scale the model by a float factor | 按浮点系数缩放模型 |
| --downward-check "machine1.json;machine2.json;..." | check whether current machine downward compatible with the machines in the list | 检查当前机器是否与列表中的机器向下兼容 |
| --export-3mf filename.3mf | Export project as 3MF. | 将项目导出为 3MF 文件 |
| --export-settings settings.json | Export settings to a file. | 将设置导出到文件 |
| --export-slicedata slicing_data_directory | Export slicing data to a folder. | 将切片数据导出到文件夹 |
| --export-stl | Export the objects as single STL. | 将对象导出为单个 STL 文件 |
| --export-stls | Export the objects as multiple STLs to directory | 将对象以多个 STL 文件的形式导出到指定目录 |
| --help, -h | Show command help. | 显示命令帮助信息 |
| --info | Output the model's information. | 输出模型的信息 |
| --load-defaultfila option | Load first filament as default for those not loaded | 为未加载丝材的对象加载第一个丝材作为默认值 |
| --load-slicedata slicing_data_directory | Load cached slicing data from directory | 从目录中加载缓存的切片数据 |
| --min-save option | export 3mf with minimum size. | 导出最小尺寸的 3mf 文件 |
| --mstpp time | max slicing time per plate in seconds. | 每块板的最大切片时间（秒） |
| --mtcpp count | max triangle count per plate for slicing. | 每块板用于切片的最大三角形数量 |
| --no-check | Do not run any validity checks, such as gcode path conflicts check. | 不运行任何有效性检查，如 G 代码路径冲突检查 |
| --normative-check option | Check the normative items. | 检查规范性项目 |
| --pipe pipename | Send progress to pipe. | 将进度发送到管道 |
| --slice option | Slice the plates: 0-all plates, i-plate i, others-invalid | 对板进行切片：0 - 所有板，i - 第 i 块板，其他 - 无效 |
| --uptodate | Update the configs values of 3mf to latest. | 将 3mf 文件的配置值更新到最新 |

### stl切片命令

./OrcaSlicer_Linux_V2.2.0.AppImage --load-settings "orca_set/machine.json;orca_set/process.json" --load-filaments "orca_set/filament.json" --slice 0 --debug 2 --orient 1 --arrange 1 --convert-unit --outputdir ./out_neza ../lixw/neza.stl

## 配置文件

### 机器配置

Creality Ender-3 V3

[machine_creality_ender_3_v3.json]

### 耗材配置

Creality Generic PLA @Ender-3 V3

[filament_Creality_Generic_PLA@Ender_3_V3.json]

### 打印过程参数配置

json { "type": "process", "name": "0.20mm Standard @Creality Ender3V3SE 0.4", "from": "system", "instantiation": "true", "adaptive_layer_height": "1", "reduce_crossing_wall": "0", "bridge_flow": "0.95", "bridge_speed": "100", "brim_width": "5", "compatible_printers": [ "Creality Ender-3 V3 SE 0.4 nozzle" ], "print_sequence": "by layer", "default_acceleration": "2500", "bridge_no_support": "0", "elefant_foot_compensation": "0", "outer_wall_line_width": "0.42", "outer_wall_speed": "60", "line_width": "0.46", "infill_direction": "45", "sparse_infill_density": "15%", "sparse_infill_pattern": "crosshatch", "initial_layer_line_width": "0.46", "initial_layer_print_height": "0.2", "initial_layer_speed": "30", "gap_infill_speed": "50", "infill_combination": "0", "sparse_infill_line_width": "0.45", "infill_wall_overlap": "15%", "sparse_infill_speed": "180", "interface_shells": "0", "detect_overhang_wall": "1", "reduce_infill_retraction": "1", "filename_format": "{input_filename_base}_{filament_type[initial_tool]}_{print_time}.gcode", "wall_loops": "2", "inner_wall_line_width": "0.45", "inner_wall_speed": "90", "print_settings_id": "", "raft_layers": "0", "seam_position": "aligned", "skirt_distance": "2", "skirt_height": "2", "minimum_sparse_infill_area": "10", "internal_solid_infill_line_width": "0.42", "internal_solid_infill_speed": "180", "spiral_mode": "0", "standby_temperature_delta": "-5", "enable_support": "0", "support_filament": "0", "support_line_width": "0.38", "support_interface_filament": "0", "support_on_build_plate_only": "0", "support_top_z_distance": "0.15", "support_interface_loop_pattern": "0", "support_interface_top_layers": "2", "support_interface_spacing": "0.5", "support_interface_speed": "100%", "support_base_pattern": "rectilinear", "support_base_pattern_spacing": "2.5", "support_speed": "50", "support_threshold_angle": "30", "support_object_xy_distance": "60%", "detect_thin_wall": "1", "top_surface_line_width": "0.42", "top_surface_speed": "50", "travel_speed": "150", "enable_prime_tower": "0", "prime_tower_width": "60", "xy_hole_compensation": "0", "xy_contour_compensation": "0", "inherits": "fdm_process_creality_common", "max_travel_detour_distance": "0", "bottom_surface_pattern": "monotonic", "bottom_shell_layers": "4", "bottom_shell_thickness": "0", "brim_object_gap": "0.1", "compatible_printers_condition": "", "top_surface_acceleration": "2500", "draft_shield": "disabled", "enable_arc_fitting": "0", "wall_infill_order": "inner wall/outer wall/infill", "initial_layer_acceleration": "500", "travel_acceleration": "2500", "inner_wall_acceleration": "2000", "ironing_flow": "15%", "ironing_spacing": "0.1", "ironing_speed": "20", "ironing_type": "no ironing", "layer_height": "0.2", "overhang_1_4_speed": "0", "overhang_2_4_speed": "20", "overhang_3_4_speed": "15", "overhang_4_4_speed": "10", "skirt_loops": "0", "resolution": "0.012", "support_type": "normal(auto)", "support_style": "grid", "support_interface_bottom_layers": "-1", "tree_support_branch_angle": "40", "tree_support_wall_count": "2", "top_surface_pattern": "monotonicline", "top_shell_layers": "4", "top_shell_thickness": "0.8", "initial_layer_infill_speed": "80", "wipe_tower_no_sparse_layers": "0", "setting_id": "GP004", "brim_type": "no_brim", "outer_wall_acceleration": "1000", "support_interface_pattern": "auto" }

添加树状支持

JSON { "enable_support": "1", "support_type": "tree(auto)", "support_style": "default" }